Vulnerability of Remote Area Infrastructure and Lifelines to Tropical Cyclone and Flood Hazards

# David King

Director of the Centre for Disaster Studies, James Cook University

Keywords: critical infrastructure, community lifelines, resilience, cyclones, floods.

Abstract

In common with much of the world, urban and infrastructural development in the tropics has concentrated people and their settlements onto the coastal fringes where the hazards of floods and tropical cyclones (hurricanes) constitute a regular threat to safety and structures. Cyclone storm surge scenarios for coastal cities in northern Australia have demonstrated that critical infrastructure and community lifelines are particularly concentrated in the most hazardous zones. The same level of concentration of services is true for the small remote and isolated settlements in Australia’s northern outback, but critical infrastructure and lifelines are much more primitive and limited such that the communities are far more vulnerable to the hazards of the tropical wet season. Because these hazards are regular and predictable, communities have developed strategies of endurance and resilience. However the regular loss of roads, drainage, sewerage, airstrips, power, water supply, restricted food supply, and access to education and health facilities makes these remote communities critically vulnerable during the wet season, and curtails economic development as resources are regularly directed to repairing losses. These characteristics of northern Australia are shared by the equally remote and isolated developing communities of the South Pacific, where the loss of the most basic infrastructure exacerbates vulnerability and secondary disaster crises.

Introduction

Throughout the world, hazard vulnerability has increased as a consequence of population growth and large numbers of people moving into coastal lowlands. In particular there has been an enormous growth in hazard prone tropical and subtropical coastal areas.  Within the developing world this process is involving extensive urbanisation -- the proliferation of many new centres and the unprecedented growth of great industrial cities.  In developed countries such as Australia urban growth has mainly been in subtropical areas, but urbanisation during the latter half of the 20th century consisted of an emptying of much of the interior of the country. While more densely populated coastal areas have experienced a concentration of people and critical infrastructure into flood, cyclone and storm surge prone hazardous areas, the remaining remote centres both in the interior and around the sparsely populated northern coasts are vulnerable to the same hazards, but are sustained by the most basic infrastructure and lifelines. This paper examines these two connected issues of hazard prone critical infrastructure and the lack of all but the most basic lifelines and infrastructural support in remote and isolated small towns and communities.

Critical infrastructure in hazard zones

Most of the population of the tropical north of Australia is concentrated in five small cities (four of them on the coast of Queensland).  Outside these cities most of the rest of the population lives in small service centres that are either rural oriented or mining towns.  Almost all of them contain less than 20,000 people.  The majority have populations of a few thousand, and while a population of 1000 is the census threshold for the definition of a town there are many places in northern Australia that have just a few hundred people and provide basic urban services to the surrounding rural population and travellers and tourists.

Many of the places of Northern Australia are indigenous settlements with only a small non-indigenous service population. Other places are ‘open’ towns where the indigenous population is often significant or in the majority. The indigenous communities have their own councils and were funded through the Commonwealth Government Aboriginal and Torres Strait Island Commission, while the open towns are part (most are headquarters) of a larger rural shire that is reliant on a small rates base for much of its funding of infrastructure.

It is a relatively new network of settlements in which places are less than 150 years old with the communication network mostly constructed in the last 70 years. Towns and cities display most of their original settlement priorities, where the centre still contains much of the critical infrastructure at the point of entry and foundation -- the port and seafront.

During the late 1990s Cairns in far North Queensland was the focus of a multi hazard assessment. A classification and mapping of all buildings was linked to a Digital Elevation Model thus giving each structure an individual elevation (Zerger 1996).  Subsequently several other towns received a similar focus.  This level of detail enabled scenarios of storm surge impact to be constructed through a GIS (Granger et al 1999).  An example of one such scenario in Cairns is illustrated by table 1.  This estimates the number of buildings that would experience seawater inundation (surge height alone, without wave set-up).  The height is above AHD (Australian Height Datum), which is approximately a mid tidal level, about 1 metre below the high water level.  Thus the example scenario illustrated in table 1 of a 3 m storm surge is a probable outcome of the severe cyclone, category three or four.  In such an event there is an equal chance that the inundation could be greater or less than the impact indicated, depending on the state and the tide.

The scenario of a 3 m storm surge is proportionately similar to other scenarios that were measured at 1, 2, 4 and 5 metre heights. At each height increment the numbers of buildings expected to be inundated increased in a similar ratio of residential to non-residential buildings.  The coastal plain below 5 m slopes consistently inland away from the coast.  The GIS recorded every building, its use, construction characteristics and materials, and assigned to each structure its land height above AHD as well as the height of the building’s lowest floor above the land.

In each scenario of 1, 2 and 3 metres above AHD, the number of residential buildings increased gradually while commercial, lifeline, and critical infrastructural buildings increased much more rapidly to the state where most would be inundated by a 3 metre surge.  However, at this scenario level only 20% of residential buildings would be inundated.

Table 1. Cairns: Impact Scenario of Storm Surge Vulnerability of Buildings Below 3 Metres above AHD

| Type of Building | Total Buildings | Buildings below 3 ms | Percent below 3 metres |
| --- | --- | --- | --- |
| Accommodation - motels, hotels etc | 267 | 162 | 61 |
| Private Houses and Flats | 30529 | 6164 | 20 |
| Number of Timber and Fibro houses |  | 2559 |  |
| Sub Total Housing | 30796 | 6326 | 20 |
| Business and commercial premises | 1533 | 1259 | 82 |
| Industry - factories etc | 79 | 68 | 86 |
| Sheds and warehouses | 76 | 43 | 57 |
| Miscellaneous - business related | 19 | 19 | 100 |
| Sub total Business & Industry | 1707 | 1389 | 81 |
| Recreational facilities | 87 | 41 | 47 |
| Community facilities - churches, halls, libraries etc. | 210 | 111 | 53 |
| Sub total Community Buildings | 297 | 152 | 51 |
| Emergency services - police, fire, SES, etc. | 40 | 25 | 62 |
| Logistics - bulk gas, fuel, storage & transport | 395 | 322 | 82 |
| Health Services | 151 | 98 | 65 |
| Power Utilities | 25 | 22 | 88 |
| Water supply | 34 | 11 | 32 |
| Telecommunications | 32 | 14 | 44 |
| Educational Facilities | 219 | 113 | 52 |
| Government Facilities & depots | 54 | 37 | 69 |
| Sub total Service Facilities | 950 | 642 | 68 |
| Total All Buildings | 33750 | 8509 | 25 |

Source: Queensland Department of Emergency Services/ Cairns City Council; 1995 survey by Granger et al.

Table 2 shows actual proportions of residential and lifeline support buildings that were inundated or isolated by a severe flood that swamped most of Townsville in January 1998.  While these actual figures are derived from sample surveys of households and service providers, they show a similar low proportion, 15% of residential buildings impacted and a high proportion, over 50%, of critical infrastructure inundated or inaccessible.

Table 2. Townsville January 1998 Floods: Businesses, Infrastructural and Community Premises

| Statement of Inundation Impact: Survey of 115 Service & Commercial Premises in Flood Zone | Flood Affected |  | Not Affected |  | Total |
| --- | --- | --- | --- | --- | --- |
| Number of Facilities and Percent |  | % |  | % |  |
| Inaccessible all or part Saturday to Monday | 57 | 50 | 57 | 50 | 115 |
| Operations were disrupted | 28 | 52 | 23 | 43 | 54 |
| Disruption from water inside building | 58 | 50 | 54 | 47 | 115 |
| Disruption from water around buildings | 56 | 49 | 54 | 47 | 115 |
| Disruption from dependency on other disrupted facilities | 50 | 43 | 50 | 43 | 115 |
| Staff unable to reach premises | 26 | 50 | 19 | 37 | 52 |
| Customers/users unable to reach or leave premises | 21 | 40 | 13 | 25 | 52 |
| Loss of stock, equipment and plant | 74 | 65 | 39 | 34 | 114 |
| Damage and loss to storage capacity | 11 | 22 | 36 | 71 | 51 |
| Restoration of normal operations delayed | 59 | 52 | 26 | 23 | 114 |
| Residential Households: Sample | 150 | 15 | 850 | 85 | 1000 |

Source: Centre for Disaster Studies Survey

Note: Some questions were not applicable, but this column has not been included.

Critical infrastructure, and lifelines and the economic basis of towns and cities in northern Australia are concentrated in the low-lying and coastal fringe hazard zones.  After Cyclone Tracy in 1974, the rebuilding of Darwin enabled some relocation of critical infrastructure to occur, and some relocation is slowly taking place in the Queensland cities. However, the process is slow and population continues to grow in all these cities.

The problem of vulnerable critical infrastructure is not unique to northern Australia.  A whole section of the World Conference on Disaster Reduction held in Kobe in January 2005 was devoted to this issue (Buckle, Queste, Shinozuka, Tierney 2005). On the other hand, research carried out by Centre for Disaster Studies members in Phuket, Thailand immediately following the Indian Ocean tsunami (King and Gurtner, 2005) revealed rapid recovery of tourist facilities and beach areas on Phuket Island precisely because the lifelines and critical infrastructure were outside the hazard zone.  The disaster response was able to move in from an intact and functioning city.  This would not be the experience of most northern Australian towns in the event of a major disaster.

Small and Remote Centres

Between 1997 and 2000 the Centre for Disaster Studies carried out post disaster appraisals in six remote towns of northern Australia.  These places and the hazards they experienced were as follows.

- Flooding of Cloncurry, a mining town in North Queensland following a major inundation of the town by its adjacent river in 1997.

- Extensive flooding of the Gulf of Carpentaria lowlands in 1998 that resulted in isolation and inundation of Burketown, Normanton, and Karumba.  Karumba is a fishing port and both Burketown and Normanton have predominantly indigenous populations.

- Wujal Wujal, an indigenous community that is also a service centre for an equal number of alternative lifestyle non-indigenous people on the rugged rainforested coastal mountains at the base of Cape York peninsula, was isolated from several weeks after Cyclone Rona passed close by in 1999.

- Broome, a tourist centre on the Kimberley coast of Western Australia was hit by severe cyclone Rosita in 2000.

In all of these events the remoteness of these places constrained response and recovery and exacerbated secondary problems such as food shortage, health crises and the use of scarce community facilities. All of the small towns and communities of northern Australia would experience loss of lifelines and critical infrastructure in a natural disaster.  Added to this vulnerability are problems associated with their small size and remoteness.

These issues of vulnerability are:

1. Small populations that impose limitations on the range of services available.

2. Following from the small population, critical infrastructure is extremely basic and many services enjoyed by city populations are entirely absent.

3. Distances of many of these small centres from their nearest neighbours and cities are vast.

4. These vast distances are linked by extremely tenuous road links, some still unsealed that are easily damaged or washed out during natural hazard events.  Remote centres are often isolated for many weeks following floods or cyclones.  Airstrips are equally vulnerable and also become inaccessible for shorter periods after a hazard.

5. Many of the people living in remote Centres are indigenous, unemployment is very high, and most households are poor.  Stockpiling supplies of food, fuel etc is not an option the most households.

6. Most shops in remote Centres are small operations dependent on the road network and are unable to carry large stocks of food. Small centres run out of food during the flood or cyclone aftermath.

7. Water supplies, power and sewerage are extremely basic services in remote centres.  These services are frequently disrupted, resulting in secondary health crises.

Table 3. Small and Remote Towns of North Queensland: Total and Indigenous Population

|  | Total Population | Aboriginal Population | Torres Strait Islanders | % Unemployed |
| --- | --- | --- | --- | --- |
| Aurukun | 778 | 689 | 3 | 2.1 |
| Bamaga | 756 | 20 | 550 | 7.7 |
| Ravenshoe | 406 | 116 | 0 | 32.5 |
| Burketown | 220 | 82 | 0 | 3.7 |
| Camooweal | 258 | 119 | 0 | 15.6 |
| Charters Towers | 8893 | 564 | 51 | 8.8 |
| Chillagoe | 193 | 52 | 0 | 14.6 |
| Cloncurry | 2459 | 583 | 8 | 9.2 |
| Collinsville | 2021 | 36 | 0 | 20.5 |
| Cooktown | 1411 | 167 | 0 | 11.3 |
| Croydon | 223 | 88 | 0 | 17.0 |
| Dajarra | 203 | 168 | 3 | 11.5 |
| Dimbulah | 429 | 19 | 6 | 13.4 |
| Daintree | 142 | 9 | 0 | 5.7 |
| Georgetown | 298 | 9 | 0 | .0 |
| Glenden | 1329 | 24 | 0 | 4.6 |
| Herberton | 994 | 69 | 40 | 16.3 |
| Hughenden | 1444 | 141 | 3 | 6.7 |
| Injinoo | 337 | 82 | 111 | 6.0 |
| Julia Creek | 519 | 36 | 0 | 3.3 |
| Karumba | 1043 | 12 | 0 | 10.9 |
| Kowanyama | 912 | 814 | 3 | .8 |
| Lockhart River | 504 | 448 | 6 | 2.5 |
| Doomadgee | 754 | 649 | 0 | 17.9 |
| Napranum | 777 | 541 | 137 | 5.9 |
| Normanton | 1328 | 640 | 20 | 9.7 |
| Pentland | 200 | 14 | 0 | 21.7 |
| Pormpuraaw | 84 | 6 | 0 | 9.8 |
| Richmond | 733 | 72 | 8 | 4.9 |
| Seisia | 276 | 133 | 56 | .0 |
| Thursday Island | 2483 | 53 | 1440 | 8.2 |
| Umagico | 231 | 99 | 101 | 7.1 |
| Weipa | 2200 | 68 | 68 | 1.3 |
| Wujal | 293 | 280 | 0 | .0 |

Source: Cdata96, ABS

Table 3 illustrates basic population characteristics of the small towns of northern Queensland.  The small size, indigenous population, and unemployment rates are fairly typical of most of the towns in the rest of northern Australia. The multi hazard assessment of Cairns by Granger and others (1999) developed a method for vulnerability analysis that ranked census collection districts in terms of population characteristics (derived from the census like those in table 3 above), lifelines and critical infrastructure, and hazard location and environmental characteristics. Granger has argued that the single most important vulnerability indicators are location in a hazardous location and total population. However, the very smallness of the remote centres (while reducing overall vulnerability because of a lack of population) increases the vulnerability of any individual, because such tiny population concentrations simply will not support a range of services. Meanwhile all consumable items, especially food and building materials are significantly more expensive than in coastal locations.

Granger’s vulnerability assessment was not intended to be used to rank different settlements against one another.  The divide between coastal cities and remote locations makes comparison impossible.  However a comparison of services and facilities was attempted to categorise vulnerability and development of small centres (King et al 2001).  An inventory of 103 services, facilities and settlement characteristics was assembled for 11 of the North Queensland small towns. These were compiled from local directories, council information and fieldwork. Not all facilities were actually in the places that were selected, but may sometimes be shared with other communities. Although the inventory appeared extensive, the 11 places are all open towns (indigenous communities are much more poorly served by private enterprise services), and most places have an extremely limited array of basic services. The result of relating population and services shows that there is a very high positive correlation between the number of services and the number of people in a place. The smaller the place the more constrained it is in what it can do, and therefore the more vulnerable it is as a community.

Figure 12. Relationship between Population and All Services and Functions for Eleven Selected Remote Towns of North Queensland

Note: Pearson Correlation - All Services & Functions with Population 0.977 Correlation is significant at the 0.01 level (2-tailed).

Despite problems and issues of vulnerability, the people of these remote and isolated communities are extremely resilient.  People are used to having to rely upon their own resources, and maintain close relationships with neighbours, family and friends.  Building on this existing resilience is the surest way of reducing vulnerability.  Having identified issues of vulnerability in these post disaster studies the Bureau of Meteorology collaborated with the Centre for Disaster Studies during 2004 in a major study of the ways in which remote and indigenous communities receive and perceive hazard warnings and weather information. Parallel to this study an indigenous research student worked in the remote indigenous community of Mornington Island to gather aboriginal hazard and weather knowledge, and also identified strong levels of community resilience and knowledge of country. Vulnerability is balanced to some extent by resilience. The enormous problems of distance, remoteness, and lack of development in the outback centres of northern Australia, make it difficult to reduce vulnerability to natural hazards.  Even though many places are growing in size, the remote towns and centres of the North are still very small, and there is little likelihood that lifelines, services and critical infrastructure will significantly improve in the short to medium term.  Thus it is the resilience of these communities that is their greatest asset in preparing for and mitigating against natural disaster.

Conclusion

An extensive awareness and preparedness campaign is a priority in the small and remote centres of the north to empower people and communities in order to increase their resilience to hazards. But they are also severely disadvantaged in terms of socio-economic status, and the provision of services and infrastructure. The region desperately needs economic investment, development and subsidisation to raise the generally low standard of living in many of the smallest places (predominantly mining towns would be the one exception to the general pattern of disadvantage). This is obviously not really the concern of Emergency Services, but it is a major political issue and comes down to the extent to which government, and the nation, are willing to subsidise the remote regions. However, what is of concern to emergency managers is the severe constraint that is placed on mitigation for these small and cash strapped places. A different set of rules or expectations must be applied to the small centres in the outback. They will continue to need higher per capita support for disaster reconstruction because their infrastructure is so basic. They also need far greater support in dealing with the allied public health problems that follow a natural hazard.

References

Buckle I (2005) “Enhancing the resilience of highway networks against extreme events. Session 4.8. Vulnerability of modern societies towards natural disasters – the impact on critical infrastructures”, Kobe Report of thematic segment. WCDR. http://www.unisdr.org/wcdr/

Granger K, Jones T & Scott, G. Editors (1999) Community Risk in Cairns: a multi-hazard risk assessment, Cities Project, AGSO, Canberra.

King D, Cottrell A, Anderson-Berry L, Cunliffe S, MacGregor C, McLachlan E, and Antrobus J (2001) Cyclone and Natural Hazard Vulnerability in Remote and Indigenous Communities of North Queensland: Final Reports. Centre for Disaster Studies, James Cook University. http://www.tesag.jcu.edu.au/CDS/index.shtml

King D and Gurtner Y, (2005) “After the Wave: A Wake Up Warning for Australian Coastal Locations”. Australian Journal of Emergency Management, Vol. 20 No. 1

Queste A (2005) “Vulnerability of modern societies towards natural disasters - the impact on critical infrastructures. Session 4.8. Vulnerability of modern societies towards natural disasters – the impact on critical infrastructures”, Kobe Report of thematic segment. WCDR. http://www.unisdr.org/wcdr/

Shinozuka M (2005) “Enhancing the resilience of power grids against extreme events. Session 4.8. Vulnerability of modern societies towards natural disasters – the impact on critical infrastructures”, Kobe Report of thematic segment WCDR. http://www.unisdr.org/wcdr/

Tierney K (2005) “Response and recovery in the perspective of enhancing the resilience of communities against extreme events. Session 4.8. Vulnerability of modern societies towards natural disasters – the impact on critical infrastructures”, Kobe Report of thematic segment. WCDR. http://www.unisdr.org/wcdr/

Zerger A (1996) “Application of Spatial Analysis and GIS for Modelling Risk in Storm Surge Prone Areas of Northern Queensland.” In: Heathcote R, Cuttler C. & Koetz J. Editors NDR96, Conference on Natural Disaster Reduction, Surfers Paradise.

Acknowledgements

Sponsored by the Commonwealth Government through the Emergency Management Australia (EMA) IDNDR Projects Program, and post disaster study scheme, and support from the Bureau of Meteorology, and infrastructure survey research funded by the Savanna Cooperative Research Centre.

Author Biography

Associate Professor David King has been the Director of the Centre for Disaster Studies at James Cook University in Townsville and Cairns in Northern Australia, since 1996, and is a corporate member of the Planning Institute of Australia.